Martin Jaime

CPE301 – SPRING 2016

# Design Assignment 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL CODE OF TASK 2 |  |  |
| 3. | INCREMENTAL CODE OF TASK 3 |  |  |
| 4. | INCREMENTAL CODE OF TASK 4 |  |  |
| 5. | INCREMENTAL CODE OF TASK 5 |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | Github LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

- Atmega328P

- Pololu USB AVR Programmer with USB mini cable

- LEDs

- 100 kΩ resistors

- 10-bit LED block

- Breadboard

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

Design a delay subroutine to generate a waveform on PORTC.0 with 50% DC and 0.5 sec period.

Assembly code:

.macro		initstack

ldi		r16,	high(ramend)

out		sph,	r16

ldi		r16,	low(ramend)

out		spl,	r16

.endmacro

initstack

sbi		ddrc,	0	;; Set first pin as output

ldi		r17,	0

ldi		r16,	0x01

out		portc,	r16	;; Set PC0 high.

start:

call		delay

eor		r17,	r16	;; toggle value written to PC0

out		portc,	r17	;; output toggled value to PC0

rjmp	start			;; Restart loop

;; Delay routine that delays for a 0.25 s

delay:

;; Set timer/counter1 to 0x85EE

push		r20		;; store r20 and restore before return.

ldi		r20,	high(34286)

sts		tcnt1h,	r20

ldi		r20,	low(34286)

sts		tcnt1l,	r20	; tcnt1 = 34286

ldi		r20,	0x00	;

sts		tccr1a,	r20	; wgm11:10 = 0x00. Normal operation

ldi		r20,	0x03

sts		tccr1b,	r20	; cs1 = 0x03. Prescaler = 64

again:

in		r20,	tifr1	; read tifr

sbrs		r20,	tov1	; if tov1 is set, skip next instruction

rjmp		again

ldi		r20,	0x00

sts		tccr1b, r20	; stop timer1. Resetting CS1

ldi		r20,	0x01

out		tifr1,	r20	; clear tov1 flag

pop		r20

ret

C code:

#define F_CPU 8000000UL //XTAL = 8 MHz

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC = 1<<DDC0;	// Set PC0 as an output

PORTC = 1<<PORTC0;	// Set PC0 high

while (1)

{

_delay_ms(250);	// delay for 0.25 s

PORTC ^= 1<<PORTC0;	// toggle PC0

}

}

| 2. | INCREMENTAL CODE OF TASK 2 |  |  |
| --- | --- | --- | --- |

Implement an 8-bit counter to count on every rising edge of the above waveform. The state of the counter needs to be displayed (display 8 bits only) on a 10-bit LED bar connected to PORTB. Do not worry about the counter overflow.

Assembly code:

.def	zero	= r0

.macro		initstack

ldi		r16,	high(ramend)

out		sph,	r16

ldi		r16,	low(ramend)

out		spl,	r16

.endmacro

initstack					;; initiate stack.

sub		zero,	zero	;; make zero register.

ldi		r20,	0	;; initiate counter.

;	sbi		ddrc,	0	;; make PC0 an output.

ldi		r16,	0xFF	;;

out		ddrb,	r16	;; make all of portB an output.

ldi		r17,	0

ldi		r16,	0x01

;	out		portc,	r16	;; output initial value of 1.

start:

call	delay

eor		r17,	r16	;; toggle output value.

sbrc	r17,	0		;; if (r17 == 1)

inc		r20				;;    r20++

;	out		portc,	r17	;; output 1/0

out		portb,	r20	;; output count to portB

rjmp	start

;; Delay routine that delays for a 0.25 s

delay:

;; Set timer/counter1 to 0x85EE

push		r20		;; store r20 and restore before return.

ldi		r20,	high(34286)

sts		tcnt1h,	r20

ldi		r20,	low(34286)

sts		tcnt1l,	r20	; tcnt1 = 34286

ldi		r20,	0x00	;

sts		tccr1a,	r20	; wgm11:10 = 0x00. Normal operation

ldi		r20,	0x03

sts		tccr1b,	r20	; cs1 = 0x03. Prescaler = 64

again:

in		r20,	tifr1	; read tifr

sbrs	r20,	tov1	; if tov1 is set, skip next instruction

rjmp	again

ldi		r20,	0x00

sts		tccr1b, r20	; stop timer1. Resetting CS1

ldi		r20,	0x01

out		tifr1,	r20	; clear tov1 flag

pop		r20

ret

C code:

#define F_CPU 8000000UL //XTAL = 8 MHz

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

unsigned char x = 1<<DDC0;		// waveform var

unsigned char counter = 0;		// counter to output

DDRB = 0xFF;				// set PORTB as output

PORTB = counter;			// output the count to PORTB

while (1)

{

_delay_ms(250);		// delay for 0.25 s

x ^= 1<<PORTC0;		// toggle x

if (x == 1)			// if x is on rising edge,

PORTB = ++counter;	// 	increment count.

}

}

| 3. | INCREMENTAL CODE OF TASK 3 |  |  |
| --- | --- | --- | --- |

Also connect the 9th and 10th bit of the LED bar to PORTC.5 and PORTC.6 pins. Toggle PORTC.5 and PORTC.6 for every 5th rising pulses and every 10th rising pulse of the counter respectively.

Assembly code:

.def	zero	= r0

.def	count5	= r25

.def	five	= r18		;; bit 5 set

.def	four	= r19		;; bit 6 set

.def	outc	= r21

.macro		initstack

ldi		r16,	high(ramend)

out		sph,	r16

ldi		r16,	low(ramend)

out		spl,	r16

.endmacro

initstack				;; initiate stack.

ldi		four,	0x10

ldi		five,	0x20

sub		zero,	zero	;; make zero register.

ldi		r20,	0	;; initiate counter.

ldi		count5,	0	;; count5 = 0.

ldi		r16,	0x30

out		ddrc,	r16	;; make PC5 and PC6 an output.

ldi		r16,	0xFF	;;

out		ddrb,	r16	;; make all of portB an output.

out		portb,	r20	;; output 0x00 to portB

ldi		r17,	0

ldi		r16,	0x01

;	out		portc,	r16	;; output initial value of 1.

start:

call		delay

eor		r17,	r16	;; toggle output value.

breq		notRising	;; if (r17 != 0)

inc		r20		;;    r20++

inc		count5

out		portb,	r20	;;	  output count to portB

cpi		count5,	5

brne		notRising

clr		count5

in		r1,	portc	;; read pinC

eor		r1,	five

out		portc,	r1

sbrc	r1,		5

rjmp	notRising

eor		r1,		four

out		portc,	r1

;	out		portc,	r17		;; output 1/0

notRising:

rjmp	start

;; Delay routine that delays for a 0.25 s

delay:

;; Set timer/counter1 to 0x85EE

push	r20		;; store r20 and restore before return.

ldi		r20,	high(34286)

sts		tcnt1h,	r20

ldi		r20,	low(34286)

sts		tcnt1l,	r20		; tcnt1 = 34286

ldi		r20,	0x00	;

sts		tccr1a,	r20		; wgm11:10 = 0x00. Normal operation

ldi		r20,	0x03

sts		tccr1b,	r20		; cs1 = 0x03. Prescaler = 64

again:

in		r20,	tifr1	; read tifr

sbrs	r20,	tov1	; if tov1 is set, skip next instruction

rjmp	again

ldi		r20,	0x00

sts		tccr1b, r20		; stop timer1. Resetting CS1

ldi		r20,	0x01

out		tifr1,	r20		; clear tov1 flag

pop		r20

ret

C code:

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1: Verify duty cycle and period: 50% duty cycle, period = 0.5 second

In assembly:

In C:

TASK 2: Implement an 8-bit counter to count on every rising edge of the above waveform. The state of the counter needs to be displayed (display 8 bits only) on a 10-bit LED bar connected to PORTB. Do not worry about the counter overflow.

Assembly Code:

The counter counts up after one period.

C code:

TASK 3: Also connect the 9th and 10th bit of the LED bar to PORTC.5 and PORTC.6 pins. Toggle PORTC.5 and PORTC.6 for every 5th rising pulses and every 10th rising pulse of the counter respectively.

For this task, I decided to use pins 4 and 5 from port C to avoid disabling the RSTDISBL fuse on the atmega328p. When I enabled it on one chip, I was not able to enter programming mode again, unless I used high voltage programming.

Assembly code:

When pin 5 is toggled.

C code:

Task 4:

| 8. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

Task 1: LED blinks with a period of 0.5 second. It is on for 0.25 s, and off for 0.25 s.

Signal output: as measured by Pololu Slo Scope

Task 2: 8-bit counter displayed on PortB through a 10-bit LED bar.

Task 3: Toggle pins 5 and 6 on port C every 5th count and every 10th count respectively. To avoid overriding the reset pin, I used pin 4 on port C instead of pin 6.

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| http:// @youtube |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| http:// @svn or github repository link |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT