Martin Jaime

CPE301 – SPRING 2016

# Design Assignment 6

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

- Atmega328P

- Pololu USB AVR Programmer

- Breadboard

- Nokia 5110 GLCD

- Atmel Studio 7

- PC

| 1. | INITIAL CODE OF TASK 1 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL		// Set clock frequency

#include <stdio.h>

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include "nokia5110.h"		// include LCD API

void ADC_init();			// Function to initialize ADC

int main(void)

{

nokia_lcd_init();		// Initialize LCD

ADC_init();			// Initialize ADC

sei();				// enable interrupts.

while(1);			// Infinite loop waiting for interrupts.

}

void ADC_init()

{

DDRC &= ~(0<<DDC0);		// Clear bit 0 of DDRC

ADMUX = 0; // use ADC0

ADMUX |= (1 << REFS0); // use AVcc as the reference

ADMUX |= (1 << ADLAR); // Right adjust for 8 bit resolution

ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0); // 128 prescale for 16Mhz

ADCSRA |= (1 << ADATE); // Set ADC Auto Trigger Enable

ADCSRB = 0; // 0 for free running mode

ADCSRA |= (1 << ADEN); // Enable the ADC

ADCSRA |= (1 << ADIE); // Enable Interrupts

ADCSRA |= (1 << ADSC); // Start the ADC conversion

}

ISR(ADC_vect)

{

uint16_t ADC_value = ADC;	// read ADC conversion

uint32_t temperature_C, temperature_F;

char *header = "Temp(F) LM35:";

char buffer[10];

// Convert ADC value to temperature in Celcius

temperature_C = (int32_t)(ADC_value * 5 * 100 / 1024.0);

// Convert Celcius to Farenheit

temperature_F = temperature_C * 1.8 + 32;

if(temperature_F < 120)

// There is noise at output that jumps to a very large value.

// This condition is to avoid that.

{

sprintf(buffer, "%u", temperature_F); // write value to buffer as string

nokia_lcd_clear();	// clear LCD

nokia_lcd_set_cursor(0, 0); // reset cursor

nokia_lcd_write_string(header,1); // write message header

nokia_lcd_set_cursor(0, 10); // move cursor down.

nokia_lcd_write_string(buffer, 3); // Write buffer.

nokia_lcd_write_string(" o", 1); // print letter 'o' as degrees symbol.

nokia_lcd_render();	// render the message to LCD.

}

_delay_ms(1000); // delay for 1 second.

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1:

Flow chart of code

| 8. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

TASK 1:

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| https://drive.google.com/folderview?id=0B4ItVBjMqlnyaVUtQjRTNkRWUjg&usp=sharing |  |  |  |
| 10. | GITHUB LINK OF THE DA |  |  |
| https://github.com/martinjaime/CpE301_Assignments2016S.git |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Martin Jaime